STANDBY

-------

A program to setup the Power Management timers for ATA-2

hard disk drives. This program is FreeWare, use it in any

way you wish. Copyright 1995 Patrick Aalto.

Disclaimer: USE THIS PROGRAM AT YOUR OWN RISK! I cannot

accept any responsibility for damages of any kind for your

using or being unable to use this program. Be aware that

a program such as this that directly accesses the registers

of your hard disk controller may potentially cause a non-

compatible hard disk controller to perform nasty things,

like low-level formatting the drive.

What, you are still reading this, after what I wrote above?

Well, okay, you don't scare easily. Lets get down to business.

If you run the program without any parameters, it just gives

you the usage info. The parameters are:

-dX    where X = device number (default = 0).

-iX    where X = idle timer value in tenths of a second.

-sX    where X = standby timer value in seconds.

Setting the idle or standby timers to 0 disable the feature.

Maximum idle value is 250 (25 seconds) and the maximum standby

value is 1250 (20 minutes 50 seconds).

For example, to set standby timer to 4 minutes for the first

hard disk, you would run:

standby -s240

The parameters are handled left to right, so you could also

say things like:

standby -d0 -i100 -s0 -d1 -s60

That would set the idle timer of the first hard disk to 10

seconds and disable it's standby mode, and set the standby

timer of the second hard disk to 1 minute.

Idle means that the spindle remains up to speed, heads are

parked over the landing zone. Drive returns to active mode

when disk access is needed. In standby mode the spindle is

at rest, otherwise it's just like idle mode.

Notes:

- The program detects whether your hard disks support Power

Management commands (many don't), and displays a message

if they don't.

- Giving a value to the standby timer causes the drive to

enter standby mode immediately, and giving a value to the

idle timer causes the drive to enter idle mode immediately.

- Many drives have idle/standby mode timers interacting, so

that setting standby timer also sets the idle timer to 5

seconds, or that setting the idle timer disables the standby

feature. You will have to experiment in which order you need

to give the parameters to get the desired effect.

- This program is incompatible with Windows 32-bit disk access

(that is, you cannot run it in a DOS window from inside

Windows). However, the timer values will naturally stay in

effect while Windows is running.

Patrick Aalto

ap@jyu.fi

http://www.jyu.fi/~ap/